# Critical Process Parameter (CPP) ↔ Critical Quality Attribute (CQA) Matrix

Document ID: PTD-03 | Revision: 4.1 | Effective: 2025-12-10

## 1. Purpose

This matrix maps process parameters to critical quality attributes and assigns a criticality rating (High / Medium / Low) based on process characterization studies, DoE, and historical multivariate analysis. Criticality reflects the strength and consistency of a parameter's effect on the attribute. The primary CQA is USP harvest yield ([harvest][Harvest] Product. Yield). Secondary/reference attributes are DSP_Final_Protein (g) and Normalized Titer by 1000.

## 2. CPP ↔ CQA Matrix

| Parameter | → Product. Yield (USP) | → Normalized Titer | → DSP_Final_Protein | Effect direction / shape |
| --- | --- | --- | --- | --- |
| [prd_final]VCD_BI | High | High | Low (via load) | Positive — primary driver |
| [Production]O2_supply_slope | High | Medium | Low | Positive — steeper ramp better |
| [flask_filtered][flask]125mL_Final Viability | High | Medium | Low | Positive — healthier seed better |
| [prd_final]pH_BI | High | Medium | Low | Negative above 6.9 (ceiling effect) |
| [media_prep][media][Media B] Osmo | High | Medium | Low | U-shape — deviation from ~310 reduces yield |
| [Production]Temp_Drop_Time_hr | High | Medium | Low | Negative — later shift lowers yield |
| [media_prep][media][Media B] Glucose_Conc | Medium | Low | Low | Over-feeding drives pH drift (interaction w/ pH) |
| [prd_final]Via_BI | Medium | Low | Low | Positive but collinear w/ seed viability |
| [prd_final]Normalized Titer by 1000_BI | Medium | — | Low | Collinear w/ VCD_BI |
| [Production]DO_Stab_Avg | Medium | Low | Low | Control health; extreme deviation harmful |
| CEO Load Cond (mS/cm) | Low | Low | Medium | Weak DSP-recovery effect |
| AEO Hold Time (hr) | Low | Low | Medium | Long holds weakly reduce recovery |
| [prd_final]pCO2_BI | Low | Low | Low | Noise in normal range |
| [prd_final]NH4+_BI | Low | Low | Low | Noise in normal range |
| [prd_final]Gln_BI | Low | Low | Low | Noise in normal range |
| [prd_final]Reactor Fill Level_BI | Low | Low | Low | Noise |
| [prd_final]Age_BI | Low | Low | Low | Noise |
| [Production]air flow_mean | Low | Low | Low | Noise |
| [harvest]Centrifuge_Bowl_Speed_mean | Low | Low | Low | Noise (clarification only) |
| [harvest]Diff_Press_Final_Filter_mean | Low | Low | Low | Noise (fouling indicator) |

## 3. Interaction Effects

pH × Glucose over-feeding: When [prd_final]pH_BI > 6.9 and [media_prep][media][Media B] Glucose_Conc is high, yield loss exceeds the sum of the individual effects. Over-feeding accelerates the pH drift above the 6.9 ceiling. This combination is flagged High criticality when co-occurring.

## 4. Interpretation Notes

Parameters rated High to Product. Yield are the levers a golden batch must control (see PTD-04). Parameters rated Low are statistical noise with respect to yield and should not be cited as root causes even if they appear in a model; they may be retained for monitoring. DSP knobs (CEO Load Cond, AEO Hold Time) affect recovery/purity (DSP_Final_Protein) but do not create product mass — consistent with the USP-vs-DSP principle in PTD-01.