# Golden Batch Analysis Report — Recommended Golden Window

Document ID: PTD-10 | Revision: 1.5 | Prepared by: MSAT | Date: 2026-05-06

Reference dataset: Campaigns 24-A through 25-C. Outcome: [harvest][Harvest] Product. Yield (mean ~76.5 g, std ~10, range ~47–99 g).

## 1. Executive Summary

Multivariate analysis of the historical campaign data identifies a reproducible "golden window" of upstream parameters that delivers top-tier USP harvest yield (≥ 85 g). The recommended window and its evidentiary basis are summarized below. This report is the exemplar conclusion the Golden Batch Multi-Agent application should reach and cite.

## 2. Recommended Golden Window

| Driver | Recommended target | Criticality | Evidence |
| --- | --- | --- | --- |
| [prd_final]VCD_BI | High (≥ 24 ×10^6 cells/mL) | High | Top mean |
| [Production]O2_supply_slope | Steep / top-quartile | High | Positive SHAP; steep ramp in all golden batches |
| [flask_filtered][flask]125mL_Final Viability | ≥ 95 % | High | Positive; healthy seed → high VCD |
| [prd_final]pH_BI | 6.7 – 6.9 (never > 6.9) | High | Threshold loss above 6.9 (see §4) |
| [media_prep][media][Media B] Osmo | ~310 (±10) mOsm/kg | High | U-shaped optimum near 310 |
| [Production]Temp_Drop_Time_hr | Early (≤ 60 hr) | High | Monotonic negative; earlier shift → higher yield |
| [media_prep][media][Media B] Glucose_Conc | 4.0 – 5.5 g/L | Medium | Over-feeding drives pH drift (interaction) |

## 3. Ranked Drivers (mean |SHAP|, yield model)

1. [prd_final]VCD_BI — strongest, positive.

2. [Production]O2_supply_slope — positive.

3. [flask_filtered][flask]125mL_Final Viability — positive.

4. [prd_final]pH_BI — negative above 6.9.

5. [media_prep][media][Media B] Osmo — U-shaped.

6. [Production]Temp_Drop_Time_hr — negative (later worse).

Collinear proxies (O2_supply_sum, Via_BI, Normalized Titer by 1000_BI) were pruned per PTD-09. Noise parameters (pCO2_BI, NH4+_BI, Gln_BI, centrifuge speed, filter DP, air-flow means) ranked low and are not part of the golden window.

## 4. Key Statistical Findings

pH threshold: batches with pH_BI > 6.9 averaged ~71.5 g vs ~78.3 g for ≤ 6.9 (Welch t-test, p ≈ 0.003). Hold pH at or below the 6.9 ceiling.

Osmolality valley: yield peaks near 310 mOsm/kg and declines for both high and low deviations (e.g., B2418 at 284 and B2506 at 331 both underperformed).

Temp-shift timing: golden batches shifted at 50–58 hr; late shifts (66–70 hr) accompanied low yield.

Interaction: high pH combined with glucose over-feeding produces excess loss beyond either factor alone (B2412).

## 5. Exemplar Batch

B2401 (92.4 g) realizes the full golden window: seed viability 97 %, VCD_BI 27.1, pH_BI 6.82, Media B Osmo 308, Temp_Drop_Time 54 hr, steep O2_supply_slope. It is the recommended reference profile.

## 6. Recommendation

Target the golden window on every batch, prioritizing the High-criticality drivers. Treat a pH excursion above 6.9, off-target osmolality, or a late temperature shift as the first-line hypotheses for any yield shortfall. Confirm all model-derived conclusions against SME process knowledge (PTD-07) and the CPP↔CQA matrix (PTD-03) before acting — the model provides evidence, not proof of causation.